# Eval 1

Likely attack type(s): DoS / Flooding via injected dominant CAN ID 0x000
Secondary characterization: Spoofing / injection of a non-baseline ID, but the strongest behavioral pattern is flooding.

| Feature | Attack rows | Benign rows |
| --- | --- | --- |
| arbitration_id | Always 0 / 0x000 | Never 0x000 |
| dlc | Always 8 | Mixed: 1, 2, 4, 5, 7, 8 |
| payload_hex_normalized | Always 0000000000000000 | Mixed payloads |
| payload_byte_sum | Always 0 | Variable |
| payload_byte_entropy_bits | Always 0 | Variable |
| payload_hamming_distance_prev_same_id | Always 0 after first same-ID occurrence | Variable |
| same_id_rate_last_time_window_hz | Median around 1176 Hz, max 1214 Hz | Max 101 Hz |
| delta_t_same_id_s | Median around 0.00082 s | Median around 0.01948 s |

The attack starts at approximately 0.476656 s. From that point onward, ID 0x000 appears repeatedly and interleaves with normal traffic. The attack ID reaches roughly 1,176 messages/s, while the fastest benign IDs peak around 101 messages/s.

This is most consistent with a CAN DoS / flooding attack because 0x000 is the highest-priority possible standard CAN identifier, the payload is constant, and the same ID is transmitted at a rate far above normal per-ID behavior.

There is no strong evidence of fuzzing, because the attack payload does not vary. There is no strong evidence of replay, because the attack ID does not appear in benign traffic. There is no strong evidence of diagnostic abuse, because the attack pattern is not tied to recognizable diagnostic request/response behavior from the available features.

General detection rule:

Attack 1: DoS / Flooding

Detect a possible CAN flooding attack when a CAN ID shows one or more of the following behaviors compared with a benign baseline:

- The ID is new or unseen in normal traffic.

- The ID has a much higher per-ID message rate than expected.

- The ID dominates a rolling time window.

- The rolling CAN ID entropy decreases because one ID becomes overrepresented.

- The payload is constant or nearly constant across repeated messages.

- The ID has unusually high CAN priority, meaning its numeric value is lower than the IDs normally seen on the bus.

This rule is explainable because flooding attacks usually involve repeated transmission of the same or similar CAN frames at abnormal frequency. If the attacker uses a very low arbitration ID, the attack can also interfere with normal bus arbitration.

# Eval2

Likely attack type(s):

Attack 1: Spoofing / injection using legitimate CAN ID 0x9F

This dataset appears to contain one distinct attack cluster.

It is not a novel-ID DoS like the previous dataset. Here, the attacker uses an ID that also exists in benign traffic: arbitration_id = 159, hexadecimal 0x9F.

The best classification is therefore:

Spoofing / injection of a legitimate CAN ID

with a secondary high-rate behavior, because the injected messages increase the normal frequency of ID 0x9F.

Evidence from features:

The dataset contains:

Total messages: 9,971
Benign messages: 8,623
Attack messages: 1,348

All attack rows have the same CAN ID and payload:

arbitration_id = 159
arbitration_id_hex = 0x9F
dlc = 8
payload_hex_normalized = 320B2F06002F3620

This ID also appears in benign traffic:

Benign ID 0x9F messages: 534
Attack ID 0x9F messages: 1,348

So the attack is not simply a new unknown ID. Instead, it abuses an existing legitimate ID.

For benign ID 0x9F, the normal payloads follow a small repeating profile of 16 values, such as:

00100040000018A1
0011004000001821
0012004000001821
...
001F0040000018A1

The attack payload:

320B2F06002F3620

does not appear in benign ID 0x9F traffic.

Timing and rate also support injection:

Attack start time: approximately 0.920600 s
Attack end time: approximately 5.247039 s

Normal pre-attack ID 0x9F same-ID rate max: 92 Hz
Attack ID 0x9F same-ID rate min: 93 Hz
Attack ID 0x9F same-ID rate median: 408 Hz
Attack ID 0x9F same-ID rate max: 428 Hz

Normal ID 0x9F appears roughly every 0.01 s, corresponding to about 100 Hz.

During the attack, the same ID appears much more frequently, with attack messages often separated by roughly 0.003 s.

This indicates that extra messages are being injected under an otherwise legitimate CAN ID.

There is no strong evidence of fuzzing, because the attack payload is constant rather than random or highly variable.

There is no strong evidence of replay, because the attack payload does not appear in the benign profile for ID 0x9F.

There is no strong evidence of diagnostic abuse from the available features.

General detection rule:

Attack 1: Spoofing / injection of a legitimate CAN ID

Detect spoofing or injection when a known CAN ID begins transmitting payloads that are not consistent with its learned benign profile, especially when this is accompanied by abnormal timing or rate behavior.

The general logic is:

- Learn a benign profile for each CAN ID.

- For each ID, learn:

  - normal payload values or payload distribution,

  - normal per-ID rate,

  - normal same-ID inter-arrival time,

  - normal payload-change behavior.

- Flag a message as suspicious when:

  - the CAN ID is known from benign traffic,

  - but the payload is new or very rare for that ID,

  - and the ID’s rate or timing behavior deviates from baseline.

- Escalate to attack when the same abnormal payload repeats over a short window.

This is explainable because legitimate CAN IDs often have stable payload structures and periodic timing. If an existing ID suddenly carries an out-of-profile payload at an abnormal rate, it is likely spoofed or injected.